## TITLE OF THE PROJECT

## SMART ALERT SYSTEM FOR HEARING IMPAIRMENTS

Team Members:

| Sl. No | Name | SRN |
| --- | --- | --- |
| 01 | N Riddhi samitha | PES1UG23AM175 |
| 02 | N V MANYA | PES1UG23AM176 |
| 03 | Navyashree J | PES1UG23AM180 |

## Components used:

- PIR Sensor

- Arduino Uno

- Sound Sensor

- 4*4 matrix keypad

- Led

- 1000 ohm Resistor

Explanation about the project :

### Smart Alert System for Hearing Impairments

It is a smart alert system designed to assist individuals with hearing impairments by providing wireless alerts. The system integrates motion detection, sound detection, and user interaction to notify the user about certain environmental events, ensuring they are aware of potential changes or hazards in their surroundings without needing to hear sound-based alarms.

This system uses various sensors to detect events like movement or noise and provides feedback through a display screen. Additionally, the user can interact with the system via a 4x4 matrix keypad to control settings or activate different features.

- Motion Detection (PIR Sensor):

  - The PIR sensor detects motion (e.g., a person entering the room) and triggers the wristband to vibrate.

- Sound Detection (Sound Sensor):

  - The sound sensor detects loud noises (e.g., doorbell, knock) and triggers the wristband to vibrate if the noise exceeds a set threshold.

- Keypad Control:

  - The 4x4 matrix keypad allows the user to interact with the system, adjust the sound threshold, turn the alert system on/off, or change settings.

This system helps hearing-impaired individuals stay aware of their environment through vibrational alerts, providing a practical, non-hearing-based notification method.

Circuit Diagram:

Arduino Component Connections:

### Connecting Arduino to Breadboard

#### Powering the Breadboard from Arduino

- Arduino 5V → Breadboard Red Power Rail (+)

- Arduino GND → Breadboard Blue/Black Ground Rail (-)

### LED Wiring

- Anode (+, Long Leg) → Breadboard

- 220Ω Resistor → Digital Pin 5 on Arduino

- Cathode (-, Short Leg) → Breadboard (Ground)

### Matrix Keypad Wiring

- Rows → Digital Pins 6, 7, 8, 9

- Columns → Digital Pins 10, 11, 12, 13

##### PIR Sensor Wiring

- VCC → Breadboard (+)

- GND → Breadboard (-)

- OUT → Digital Pin 2 on Arduino

### Sound Sensor Wiring

- VCC → Arduino 5V

- GND → Arduino GND

- DO → Digital Pin 3 on Arduino

Pseudocode:

- Start‑up (runs once)

  - Tell the Arduino which pins are connected to:

    - the motion sensor (PIR)

    - the LED

    - the sound sensor

    - the 4 × 4 keypad

  - Open the USB/Serial connection so you can read messages on your computer.

  - Print “System Ready” to show everything is set up.

- Endless loop (runs over and over)
 Every pass through the loop the board asks three simple questions:

  A. “Did I see motion?”

  - Look at the PIR motion‑sensor pin.

  - If it’s HIGH (means movement was detected):

    - Print “Motion Detected!” on the computer screen.

    - Turn the LED on for half a second so someone nearby also sees a flash.

    - Turn the LED off again.

B. “Did I hear a loud sound?”

  - Check the sound‑sensor pin.

  - If it’s HIGH (means a clap, shout, knock, etc. was heard):

    - Print “Sound Detected!” on the screen.

    - Wait half a second (gives the sensor a moment to settle).

C. “Did someone press a keypad button?”

  - Ask the keypad object for the most recent key that was pressed.

  - If a key was pressed:

    - Show “Key Pressed: X” (X is the key) on the screen.

    - If that key is a number 0‑9:

      - Look up a canned phrase in a list:
 0→Hello, 1→I need help, … 9→Goodbye

      - Print that phrase on the screen.

    - If the key is anything else (A, B, C, D, *, #):

      - Print “Invalid Key”.

    - Pause 2 seconds so rapid key hits don’t flood the screen.

And then the loop starts again, forever, watching for motion, sound, or keypad presses.

Code:

#include <Wire.h>

#include <Keypad.h>

// Sensor pins

const int pirPin = 2;

const int ledPin = 5;

const int soundSensorPin = 3;  // Sound sensor digital output

// Keypad setup

const byte ROWS = 4;

const byte COLS = 4;

char keys[ROWS][COLS] = {

{'1', '2', '3', 'A'},

{'4', '5', '6', 'B'},

{'7', '8', '9', 'C'},

{'*', '0', '#', 'D'}};

byte rowPins[ROWS] = {6, 7, 8, 9};

byte colPins[COLS] = {10, 11, 12, 13};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

// Messages for keypad inputs

String messages[10] = {

"Hello", "I need help", "How are you", "I am hard of hearing",

"Bathroom", "Emergency", "I am fine", "Thank you",

"Where are you?", "Goodbye"};

void setup() {

pinMode(pirPin, INPUT);

pinMode(ledPin, OUTPUT);

pinMode(soundSensorPin, INPUT);

Serial.begin(9600);

Serial.println("System Ready");

}

void loop() {

// Check PIR sensor

if (digitalRead(pirPin) == HIGH) {

Serial.println("Motion Detected!");

digitalWrite(ledPin, HIGH);

delay(500);

digitalWrite(ledPin, LOW);

}

// Check sound sensor

if (digitalRead(soundSensorPin) == HIGH) {

Serial.println("Sound Detected!");

delay(500);

}

// Check keypad input

char key = keypad.getKey();

if (key) {

Serial.print("Key Pressed: ");

Serial.println(key);

if (key >= '0' && key <= '9') {

Serial.println(messages[key - '0']);

} else {

Serial.println("Invalid Key");

}

delay(2000);

}

}

Simulation Output: